A. Accident Detected Flow

- System Initialization

  - Power is supplied to the Raspberry Pi Zero W and all sensors.

  - Sensors (accelerometer/gyroscope) are calibrated.

  - GSM and GPS modules initialize.

- Continuous Monitoring

  - The system continuously reads motion data from the accelerometer and gyroscope.

  - Multi-sensor logic (acceleration + orientation) is applied.

- Threshold Detection

  - If acceleration exceeds the programmed threshold (e.g., ≥ 3g–5g), the system flags a potential accident.

- Impact Validation

  - The system checks:

    - Sudden spike in acceleration

    - Abnormal tilt/orientation

  - If both conditions are met, it confirms a possible accident.

- Trigger Alert Sequence

  - A 5-second countdown is activated.

  - A pre-recorded voice alert plays through the speaker.

- Voice Cancellation Window

  - The system listens for voice command (e.g., “cancel”) using speech recognition.

  - If no valid command is detected within 5 seconds, proceed to next step.

- Emergency Alert Transmission

  - The SIM800L sends SMS alerts to registered contacts.

  - GPS coordinates are included in the message.

- System Logging

  - Event data (time, acceleration, location) is stored for record.

- Buzzer

  - The buzzer sounds when received any text from the Rescue Center of the Barangay.

B. False Detection Flow (No Actual Accident)

- System Initialization

  - Same as normal startup (sensor calibration and module initialization).

- Continuous Monitoring

  - Real-time motion data is continuously analyzed.

- Threshold Trigger

  - Acceleration exceeds threshold due to:

    - Sudden movement

    - Helmet drop

    - Road bump

- Preliminary Detection

  - System flags it as a potential accident.

- Countdown Activation

  - 5-second countdown begins.

  - Audio alert is played.

- User Intervention (Cancellation)

  - Rider responds by:

    - Saying “cancel” (voice command)

- Command Recognition

  - Speech recognition detects the cancellation command.

- Alert Cancellation

  - Countdown stops immediately.

  - No SMS is sent.

  - System resets to monitoring state.